MOCNESS Check List

Pre-Tow

| BUCKETS |  |
| --- | --- |
| Buckets on securely and taped? |  |
| Funnels in Buckets? |  |
| Hose Clamps tight? |  |
| NETS |  |
| Nets in good shape (no rips or tears)? |  |
| Rings and shackles (trigger snaps) all attached and tight? |  |
| SENSORS |  |
| Cables plugged in all the way? |  |
| Tubing removed from Cond. Probe? |  |
| Flow meter stop removed? |  |
| FRAME |  |
| Nuts and bolts tight esp vert rod nuts? |  |
| Net bar trap in place? |  |
| TOW BRIDLE |  |
| Bridle and Figi firmly attached, signal cable plugged in, and taped down clear of shackles etc |  |
| Comments: |  |

Electrical Check-out

| Check flowmeter by letting it spin for 3 or 4 counts. Did counts register on the deck unit? |  |
| --- | --- |
| Check net command by sending a command (person on deck should watch out for falling net bar). Did toggle release step? |  |
| Check net response by depressing trip bar. Did computer beep and net number counter increment, and flow counter reset to zero? |  |
| Comments: |  |

Post -Tow

| Modem deck Unit turned off? |  |
| --- | --- |
| Tube with freshwater back on Cond. Probe? |  |
| Frame tied down and secure? |  |
| Flow meter secured? |  |
| Comments: |  |