RUNNING A MOCNESS TOW

- Start Windows.

- Double click on "MOCNESS ACQUISITION" Icon (To start Program)

- Click on "ACQUISITION SETUP"

- Select "Net Tow Info"

- Change settings for tow (only those that need changing i.e. data, haul #, etc.)

- Click "OK" if changes made ... otherwise "CANCEL".

- Click on "ACQUISITION SETUP" (again)

- Select (click) "REAL TIME ACQUISITION" (for net tow)

- You want data to be written to a file such as "c:\MOCNESS\MOCDATA".

- Double click "MOCDATA", then enter filename (e.g. m1-####.pro)

- Click on "OK" to create both the *.pro and the *.raw file or "CANCEL" to exit without file creation.

- Click on "HARDWARE SETUP".

Select "SELECT ENVIRONMENTAL SENSORS".

- Select T/C sensor "OK" if sensor #'s in boxes ok.

- Click on sensor # to select different sensor (if necessary), and then select "OK" when finished.

Select "SETUP SEA CABLE".

- This will activate the MOCNESS electronics to test the tow cable and determine the baud rate for data telemetry.  It must be done before "ACQUIRE DATA (RUN) below.

- Click on "PLOT SETUP".

- Select any option

- Set max/min time

- Set max/min latitude and longitude (in degrees and fractions of degrees)

- Set depth max/min

- etc

- Click on "RUNTIME OPTIONS"

Select "ACQUIRE DATA (RUN)" and start the station tow.

PRE-DEPLOYMENT CHECK OF THE SYSTEM

- Note bottom depth and your sampling bottom depth.

- Make up your tow profile and time table.

ON DECK

    - Have a person on deck give you:

    - Flow counts (spin flowmeter to simulate flow).

    - Net command confirmation (send 1-step command from PC).

    - Net response (depress lever on net response)

    - When all is ok, click on "RESET" command button to reset display.

    - MOCNESS can go over the side, but have the winch operator hold it just below the sea surface to stabilize sensors, etc.

- While running tow:

- Use "NET COMMAND" button (on 9-net system, both left and right on double system) to send 3-step commands to motor and drop net.

- Use "INCREMENT NET #" button if net response fails.

- Use "RESET" button to clear all net registers before a tow

- Use "PAUSE" button to temporarily stop acquisition during tow.

- At end of acquisition:

- Click on "END ACQUISITION" command button and wait for "acquisition ended" message to appear in the message box on the screen.

- Click on "ACQUISITION SETUP"

- Select "EXIT" to leave program.